DOCUMENTATION FOR THE DUAL UPPER BOUND LP ( DUBLP ) PROGRAM.

This program is based on the method of LP given in the appendix to chapter 6

of your text. The program is written in TURBO PASCAL.

To run the LP just enter DUBLP and then hit the enter key.

The program asks for three things:

DO YOU WANT DIAGNOSTIC OUTPUT? Y or N  --- To which you respond as you wish.

With the diagnostics on you get a reprint of the input file and some

information on which constraints are not yet satisfied as the solution

progresses.

DO YOU WANT OUTPUT ON THE PRINTER? Y or N -- Answer Y or y to get printer

output. Note that if you choose diagnostics you will get them on the printer

also.

ENTER NAME OF INPUT FILE :  -- Here you should enter the disk, directory and

file name as is normally done with DOS. If the file is in error or

can't be found a message will appear on the screen and the program will abort.

With or without diagnostics you then get a display of the solution progress

showing pivot variables. At the end a full variable by variable output appears

on the screen as well as on the printer if printer output was selected. As

shown below, the input file allows you to enter a label for each variable

so that the output is rather self explanatory. NOTE  a limit of 40 iterations

is built into the LP and if you reach 40 the program will stop.

INPUT FILE FORMAT:

Line 1 and Line 2  - Title lines. May be any ASCII character string up to 80

characters long each.

Line 3 -- contains two integer numbers separated by a space. The first is

the number of variables in the problem and the second is the number of

constraints.

Line 4 -- contains the cost function coefficients. These must be floating

point numbers( ie they have a decimal point.) and there must be as many of them

as there are variables. Separate them with a space.

Line 5 -- contains the cost function constant. Often the cost functions are of

the type   C1(X1) + C2(X2) + ...

where C1(X1) = A1 + B1*X1

C2(X2) = A2 + B2*X2

etc.

the cost function for the LP then must consist of:

(A1+A2+...) + B1*X1 + B2*X2 + ...

The first term in this function, ie (A1 + A2 + ...) is placed into line 5 and

the B1, B2, ... are placed into line 4.

Lines 6,7 ...  -- Contains one line for each constraint. Each line consists of

a row from the constraint tableau, that is, it contains the constraint

coefficients for the constraint - one for each variable. This is followed by

one of the symbols: = < or > to signify the type of constraint.(DO NOT put

single quotes or double quotes around these symbols.) finally the constraint

right hand side or limit is entered as a floating point number.

The last group of lines contains the variable limits. Each line contains the

variable number followed by the variable min and max and then followed by a

variable label of up to 40 characters long.

The example below shows the file used to solve the fuel scheduling problem in

example 6E.

<- indicates comments only.

base case data for problem in section 6.6 of text

--------------------------------------------------

20 12    <- Num of variables = 20, num of constraints = 12

<----record below is the cost function

0.0    0.0    0.0    1811.0 0.0    1789.0 0.0    0.0    1811.0 0.0

0.0    1789.0 0.0    0.0    1811.0 0.0    0.0    1789.0 0.0    0.0

633250.8  <---- cost ftn constant

1.0    0.0    1.0   -60.4   0.0    0.0    -1.0   0.0    0.0    0.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 2775.4

0.0    1.0    0.0    0.0    1.0   -59.7   0.0    0.0    0.0   -1.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 4260.5

0.0    0.0    0.0    0.0    0.0    0.0    1.0    1.0   -60.4   0.0

0.0    0.0   -1.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 2775.4

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    1.0

1.0   -59.7   0.0    0.0    0.0   -1.0    0.0    0.0    0.0    0.0   = 4260.5

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0

0.0    0.0    1.0    1.0   -60.4   0.0    0.0    0.0   -1.0    0.0   = 2775.4

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0

0.0    0.0    0.0    0.0    0.0    1.0    1.0   -59.7   0.0   -1.0   = 4260.5

0.0    0.0    1.0    0.0    1.0    0.0    0.0    0.0    0.0    0.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 40000.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    1.0    0.0    0.0

1.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 40000.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0

0.0    0.0    0.0    1.0    0.0    0.0    1.0    0.0    0.0    0.0   = 40000.0

0.0    0.0    0.0    1.0    0.0    1.0    0.0    0.0    0.0    0.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 1200.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    1.0    0.0

0.0    1.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0   = 1500.0

0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0

0.0    0.0    0.0    0.0    1.0    0.0    0.0    1.0    0.0    0.0   = 800.0

1  70000.0   70000.0    volume coal pile 1 week 1  <---- variable 1 limits

2  70000.0   70000.0    volume coal pile 2 week 1

3      0.0   40000.0    coal delivery to plant 1 week 1

4    150.0     600.0    plant 1 MW week 1

5      0.0   40000.0    coal delivery to plant 2 week 1

6    400.0    1000.0    plant 2 MW week 1

7      0.0  200000.0    volume coal pile 1 week 2

8      0.0   40000.0    coal delivery to plant 1 week 2

9    150.0     600.0    plant 1 MW week 2

10     0.0  200000.0    volume coal pile 2 week 2

11     0.0   40000.0    coal delivery to plant 2 week 2

12   400.0    1000.0    plant 2 MW week 2

13     0.0  200000.0    volume coal pile 1 week 3

14     0.0   40000.0    coal delivery to plant 1 week 3

15   150.0     600.0    plant 1 MW week 3

16     0.0  200000.0    volume coal pile 2 week 3

17     0.0   40000.0    coal delivery to plant 2 week 3

18   400.0    1000.0    plant 2 MW week 3

19     0.0  200000.0    volume in coal pile 1 week 4

20     0.0  200000.0    volume in coal pile 2 week 4 <---Variable 2 limits

The disk contains these files:

DUBLP.PAS Is the PASCAL source code for the LP.

DUBLP.EXE Is the LP executable file.

DUBLP.DOC Is this documentation file.

LPTEST.DAT This data file contains the LP data for the problem worked out in

the appendix to chapter 6.

EX6E-1.DAT Contains the data file above for the fuel scheduling problem in

example 6E.

EX6E-2.DAT, EX6E-3.DAT are data files for the variations shown in example 6E.

A sample run of DUBLP looks like:

DO YOU WANT DIAGNOSTIC OUTPUT ? (Y or N) Y <-- User chooses to see diagnostics

DO YOU WANT OUTPUT ON PRINTER? (Y or N) Y <-- Output will go to printer

ENTER NAME OF INPUT DATA FILE : EX6E-1.DAT

############################# This output appear on printer ##################

file: ex6e-1.dat

base case data for problem in section 6.6 of text ( altered to inc init V's )

-----------------------------------------------------------------------------

Number of variables = 20

Numbar of constraints = 12

Objective function

0.0E+00  0.0E+00  0.0E+00  1.8E+03  0.0E+00

1.8E+03  0.0E+00  0.0E+00  1.8E+03  0.0E+00

0.0E+00  1.8E+03  0.0E+00  0.0E+00  1.8E+03

0.0E+00  0.0E+00  1.8E+03  0.0E+00  0.0E+00

Constraint 1

1.0E+00  0.0E+00  1.0E+00  -6.0E+01  0.0E+00

0.0E+00  -1.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   2.8E+03

Constraint 2

0.0E+00  1.0E+00  0.0E+00  0.0E+00  1.0E+00

-6.0E+01  0.0E+00  0.0E+00  0.0E+00  -1.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   4.3E+03

Constraint 3

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  1.0E+00  1.0E+00  -6.0E+01  0.0E+00

0.0E+00  0.0E+00  -1.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   2.8E+03

Constraint 4

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  1.0E+00

1.0E+00  -6.0E+01  0.0E+00  0.0E+00  0.0E+00

-1.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   4.3E+03

Constraint 5

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  1.0E+00  1.0E+00  -6.0E+01

0.0E+00  0.0E+00  0.0E+00  -1.0E+00  0.0E+00     =   2.8E+03

Constraint 6

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

1.0E+00  1.0E+00  -6.0E+01  0.0E+00  -1.0E+00     =   4.3E+03

Constraint 7

0.0E+00  0.0E+00  1.0E+00  0.0E+00  1.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   4.0E+04

Constraint 8

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  1.0E+00  0.0E+00  0.0E+00

1.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   4.0E+04

Constraint 9

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  1.0E+00  0.0E+00

0.0E+00  1.0E+00  0.0E+00  0.0E+00  0.0E+00     =   4.0E+04

Constraint 10

0.0E+00  0.0E+00  0.0E+00  1.0E+00  0.0E+00

1.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   1.2E+03

Constraint 11

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  1.0E+00  0.0E+00

0.0E+00  1.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00     =   1.5E+03

Constraint 12

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  0.0E+00

0.0E+00  0.0E+00  0.0E+00  0.0E+00  1.0E+00

0.0E+00  0.0E+00  1.0E+00  0.0E+00  0.0E+00     =   8.0E+02

1  7.0000000000E+04  7.0000000000E+04

2  7.0000000000E+04  7.0000000000E+04

3  4.0000000000E+04  0.0000000000E+00

4  6.0000000000E+02  1.5000000000E+02

5  4.0000000000E+04  0.0000000000E+00

6  1.0000000000E+03  4.0000000000E+02

7  2.0000000000E+05  0.0000000000E+00

8  4.0000000000E+04  0.0000000000E+00

9  6.0000000000E+02  1.5000000000E+02

10  2.0000000000E+05  0.0000000000E+00

11  4.0000000000E+04  0.0000000000E+00

12  1.0000000000E+03  4.0000000000E+02

13  2.0000000000E+05  0.0000000000E+00

14  4.0000000000E+04  0.0000000000E+00

15  6.0000000000E+02  1.5000000000E+02

16  2.0000000000E+05  0.0000000000E+00

17  4.0000000000E+04  0.0000000000E+00

18  1.0000000000E+03  4.0000000000E+02

19  2.0000000000E+05  0.0000000000E+00

20  2.0000000000E+05  0.0000000000E+00

ITER = 0  COST =   2.9617500000E+06

constraint 1 violation =   5.8164600000E+04

constraint 2 violation =   4.1859500000E+04

constraint 3 violation =   1.1835400000E+04

constraint 4 violation =   2.8140500000E+04

constraint 5 violation =   1.1835400000E+04

constraint 6 violation =   2.8140500000E+04

constraint 7 violation =   4.0000000000E+04

constraint 8 violation =   4.0000000000E+04

constraint 9 violation =   4.0000000000E+04

constraint 10 violation =   6.5000000000E+02

constraint 11 violation =   9.5000000000E+02

constraint 12 violation =   2.5000000000E+02

pivot row =    1   pivot col =    1

ITER = 1  COST =   3.5950008000E+06

constraint 1 violation =   5.8164600000E+04

constraint 2 violation =   4.1859500000E+04

constraint 3 violation =   1.1835400000E+04

constraint 4 violation =   2.8140500000E+04

constraint 5 violation =   1.1835400000E+04

constraint 6 violation =   2.8140500000E+04

constraint 7 violation =   4.0000000000E+04

constraint 8 violation =   4.0000000000E+04

constraint 9 violation =   4.0000000000E+04

constraint 10 violation =   6.5000000000E+02

constraint 11 violation =   9.5000000000E+02

constraint 12 violation =   2.5000000000E+02

pivot row =    1   pivot col =    7

ITER = 2  COST =   3.5950008000E+06  <-- This will go up as feasibility

is approached

constraint 2 violation =   4.1859500000E+04

constraint 3 violation =   4.6329200000E+04

constraint 4 violation =   2.8140500000E+04

constraint 5 violation =   1.1835400000E+04

constraint 6 violation =   2.8140500000E+04     <-- Constraints will all

constraint 7 violation =   4.0000000000E+04          be satisfied at end

constraint 8 violation =   4.0000000000E+04

constraint 9 violation =   4.0000000000E+04

constraint 10 violation =   6.5000000000E+02

constraint 11 violation =   9.5000000000E+02

constraint 12 violation =   2.5000000000E+02

pivot row =    3   pivot col =    1

etc.

etc.

ITER = 25  COST =   6.9134508000E+06

SOLUTION

XMIN         X            XMAX       VARIABLE DESCRIPTION

1   70000.0000   70000.0000   70000.0000  volume coal pile 1 week 1

2   70000.0000   70000.0000   70000.0000  volume coal pile 2 week 1

3       0.0000       0.0000   40000.0000  coal delivery to plant 1 week 1

4     150.0000     200.0000     600.0000  plant 1 MW week 1

5       0.0000   40000.0000   40000.0000  coal delivery to plant 2 week 1

6     400.0000    1000.0000    1000.0000 plant 2 MW week 1

7       0.0000   55144.6000  200000.0000 volume coal pile 1 week 2

8       0.0000   19013.5000   40000.0000  coal delivery to plant 1 week 2

9     150.0000     500.0000     600.0000  plant 1 MW week 2

10       0.0000   46039.5000  200000.0000 volume coal pile 2 week 2

11       0.0000   20986.5000   40000.0000 coal delivery to plant 2 week 2

12     400.0000    1000.0000    1000.0000 plant 2 MW week 2

13       0.0000   41182.7000  200000.0000 volume coal pile 1 week 3

14       0.0000       0.0000   40000.0000 coal delivery to plant 1 week 3

15     150.0000     150.0000     600.0000 plant 1 MW week 3

16       0.0000    3065.5000  200000.0000 volume coal pile 2 week 3

17       0.0000   40000.0000   40000.0000 coal delivery to plant 2 week 3

18     400.0000     650.0000    1000.0000 plant 2 MW week 3

19       0.0000   29347.3000  200000.0000 volume in coal pile 1 week 4

20       0.0000       0.0000  200000.0000 volume in coal pile 2 week 4