The state estimator is simulated with three programs.

1) The loadflow program simulates the power system. To use the state

estimator you must first simulate the power system by running

the loadflow program. The loadflow program requires reading in

the power system network data file and solving for the

voltages. To use the state estimator you then must write out the

voltage solution data file.( see LOADFLOW.DOC )

2) The scada program ( SCADA ) simulates the Supervisory Control And

Data Acquision system. A SCADA system consists of remote terminal

units, communication paths, master station units, and a computer

system to read data directly from the power system and transmmit

and store the measured data into the computer system where it may

be acted upon by the state estimator.

In running the scada program the user must enter the power

system network data file name, the network voltage solution data

file name and the measurement data file name. Last, the user

gives the name for the state estimator input file ( see SCADA.DOC )

3) The state estimator program (SE ) reads the state estimator input

file created by the scada program and allows the user to run the

state estimation algorithm, make printouts and alter input data.

( see SE.DOC )

PROGRAM DIMENSIONS ( Applies to LOADFLOW, SCADA AND SE )

Max number of buses = 25

Max number of branches ( lines or transformers ) = 40

Max number of transformers = 10

Max number of voltage measurements = 25

Max number of angle measurements = 25

Max number of injection measurements ( P +jQ ) = 25

Max number of branch flow measurements ( P + jQ ) = 80

Total number of measurements ( voltage, angle, injection and branch flow) is

limited to 100. This means you cannot have all branches measured at each end,

plus all injections measured, plus all bus voltages measured. This results

from a limitation on the size of the data areas available in TURBO PASCAL.

To create useful exercises, you can run the loadflow and scada programs

and then alter the contents of the state estimator input file to simulate

bad measurements. All of the files used are ASCII data files and can be

edited directly.

Exercises using the state estimator are described in file STATEEST.EXR

Files having to do with the state estimator are:

STATEEST.DOC  <-- Overall document for state estimator

SCADA.DOC     <-- Scada program documentation

SCADA.EXE     <-- Scada loaded file

SCADA.PAS     <-- Scada PASCAL source file

SE.DOC        <-- State Estimator documentation

SE.EXE        <-- State estimator loaded file

SE.PAS        <-- State estimator PASCAL source file

SIXBUS.DAT    <-- Duplicate six bus base case file

SIXBUS2.DAT   <-- Six bus system with changes

SOL6-1.DAT    <-- Solution ourput for SIXBUS.DAT from LOADFLOW

SOL6-2.DAT    <-- Solution ourput for SIXBUS2.DAT from LOADFLOW

MEAS6-1.DAT   <-- Measurement data for base six bus system

MEAS6-2.DAT   <-- Measurement data for second six bus case

SE6-1.DAT     <-- State estimator input file for base six bus network

SE6-2.DAT     <-- State estimator input file for second case

STATEEST.EXR  <-- Sample exercises